AI-POWERED HYDROPONICS

PLANT GROWTH MONITORING SYSTEM

Pseudocode  •  React Native App  •  ESP32-S3 Nano  •  Roboflow AI

4-Phase Architecture: Setup  →  Data Collection  →  App Development  →  AI Monitoring

PHASE 1: Setup & Configuration Hardware setup, environment, constants, and model preparation

1.1 — System Constants

PROGRAM HydroponicsMonitoringApp:

// Hardware

CONST BOARD_TYPE    = 'Arduino Nano ESP32'

CONST CAMERA_MODULE = 'OV2640'  // external module, manual GPIO wiring

CONST LOGIC_VOLTAGE = 3.3       // NOT 5V

// OV2640 Camera Pin Definitions (ESP32-S3 Nano)

PIN XCLK=15, SIOD=4, SIOC=5, VSYNC=6, HREF=7, PCLK=13

PIN Y9=16, Y8=17, Y7=18, Y6=12, Y5=10, Y4=8, Y3=9, Y2=11

// WiFi & Server

CONST WIFI_SSID      = 'YourNetworkName'  // 2.4GHz only

CONST WIFI_PASSWORD  = 'YourPassword'

CONST SERVER_IP      = '192.168.x.x'

CONST SERVER_PORT    = 3000               // Node.js

// Roboflow AI

CONST ROBOFLOW_API_KEY  = '<your_api_key>'

CONST ROBOFLOW_MODEL_ID = 'kangkong-growth-stages'

CONST PLANT_SPECIES     = 'Kangkong'

// Growth Stage Mapping

CONST GROWTH_STAGES = { germination, seedling, vegetative, mature, overgrown, stressed }

1.2 — Hardware Wiring (OV2640 → ESP32-S3 Nano)

⚠️ IMPORTANT: ESP32-S3 Nano has NO built-in camera. Wire the OV2640 breakout board manually using 3.3V logic only. Never connect 5V to any GPIO pin.

Connect OV2640 to ESP32-S3 Nano as follows: 3.3V → 3.3V pin, GND → GND, SDA → GPIO4, SCL → GPIO5, XCLK → GPIO15, VSYNC → GPIO6, HREF → GPIO7, PCLK → GPIO13, D0–D7 → GPIO11/9/8/10/12/18/17/16. PWDN and RESET are not connected (set to -1 in code).

1.3 — Arduino IDE Board Setup

- Install ESP32 board support via Boards Manager (espressif/arduino-esp32)

- Select: Tools → Board → Arduino ESP32 Boards → Arduino Nano ESP32

- Partition Scheme: Huge APP (3MB No OTA) — required for camera frame buffer

- Upload via USB-C directly — no FTDI adapter or GPIO0 jumper needed

- Serial Monitor baud rate: 115200

1.4 — Roboflow Model Training Setup

PROCEDURE TrainRoboflowModel():

FOR EACH stage IN [germination, seedling, vegetative, mature, overgrown, stressed]:

COLLECT 50-100 plant photos at each growth stage

CAPTURE from the same angle and distance as camera mount position

UPLOAD all images TO Roboflow project

LABEL each image with the appropriate stage class

APPLY preprocessing: auto-orient, resize 640x640

APPLY augmentations: flip, brightness +-15%, blur 2px, rotation +-10deg

SELECT model: YOLOv8n

START training  // approx. 30 min - 2 hours

COPY API_KEY, MODEL_ID, VERSION into server config

END PROCEDURE

PHASE 2: Daily Photo Capture 15-day dataset collection using fixed camera + phone for training data

Overview

🎯 Goal: Collect a 15-day plant growth dataset for ML training. Two cameras are used: a fixed ESP32 camera (top-down view) and a phone (side-view). Training is done in Roboflow after collection.

2.1 — Fixed Camera (ESP32 — Top-Down View)

Placed above all three plant variations to capture top-down growth every day.

- Camera captures at 8:00 AM every day for 15 days

- Takes 3 photos every 1 hour, from 8 AM to 5 PM

- Total: ~30 photos per day per plant

- Camera enters sleep mode at 5:00 PM

- Photos are automatically saved to ESP32 memory (SD card / internal)

- Captures growth length/spread from above view for daily progress tracking

2.2 — ESP32 Capture Firmware Flow

PROCEDURE Main():  // Runs on boot or timer wakeup

CALL ConnectWiFi()

IF WiFi fails AFTER 20 attempts: DeepSleep(60s), RETURN

CALL SyncTimeNTP(UTC+8)

current_hour = GET CurrentHour()

IF current_hour >= 8 AND current_hour < 17:

CALL CaptureAndSave()  // top-down photo saved to memory

CALL DeepSleep(3600s)  // sleep 1 hour, repeat

ELSE IF current_hour >= 17:

CALL DeepSleep(until 8AM tomorrow)  // camera sleeps 5PM

END PROCEDURE

PROCEDURE CaptureAndSave():

CALL InitCamera()  // OV2640 via GPIO

CALL WarmUpCamera(5 frames)

photo = CALL CaptureJPEG()

IF photo is valid:

filename = 'plant_' + timestamp + '.jpg'

SAVE photo TO SD card or internal storage

END PROCEDURE

2.3 — Phone Camera (Side View)

Used to capture side-angle photos of each plant variation for height growth dataset.

- Capture anytime between 8 AM and 5 PM

- Take 40 photos per variation per day

- Photos show side view — used to measure plant height growth over time

- Each plant variation is photographed separately

2.4 — Dataset Summary (15 Days)

- Fixed camera: ~30 photos/day × 15 days = 450 top-down photos per plant

- Phone: 40 photos/day × 15 days = 600 side-view photos per variation

- After Day 15: dataset collection is complete

- Upload all photos to Roboflow and train the model

- After training is complete: proceed to Phase 3

PHASE 3: Creating the App Build React Native app and inject the trained Roboflow model

3.1 — App Overview

Build the mobile app using React Native and integrate the trained Roboflow model for real-time plant growth classification.

PROCEDURE BuildApp():

INITIALIZE React Native project (Expo)

CREATE screens: Home, LiveFeed, GrowthChart, History, Settings

INJECT Roboflow model (API Key + Model ID + Version)

CONFIGURE API_BASE_URL = 'http://' + SERVER_IP + ':3000'

TEST app connectivity to Node.js backend

END PROCEDURE

3.2 — App Screens

- HomeScreen — Dashboard: stats cards, health status, growth progress bar

- LiveFeedScreen — Latest captured photo + AI analysis results

- GrowthChartScreen — Line/bar chart of growth stage confidence over time

- HistoryScreen — Scrollable, filterable log of all growth entries

- SettingsScreen — API URL, plant species, notification toggles, dark mode

3.3 — App Entry Point

COMPONENT App():

RETURN:

<NavigationContainer>

<TabNavigator>

<Tab icon='home'>    HomeScreen    </Tab>

<Tab icon='camera'>  LiveFeedScreen </Tab>

<Tab icon='chart'>   GrowthChart   </Tab>

<Tab icon='history'> HistoryScreen </Tab>

</TabNavigator>

</NavigationContainer>

END COMPONENT

✅ Milestone: Trained Roboflow model is injected into the app. App is built and tested. Ready for Phase 4.

PHASE 4: AI Image Analysis & Growth Monitoring New plants added — ongoing AI-powered growth tracking every 2 days

4.1 — New Plant Registration

PROCEDURE AddNewPlant():

REGISTER new plant entry with: species, date_planted, device_id

SET observation_start = today

SET observation_active = TRUE

END PROCEDURE

4.2 — Recurring Photo Capture (Every 2 Days)

Once a new plant is added, photos are taken every 2 days for AI-based stage classification and growth prediction.

PROCEDURE MonitorPlantGrowth():

WHILE observation_active:

// Capture 5 photos every 2 days

IF days_since_last_capture >= 2:

FOR i = 1 TO 5:

photo = CALL CaptureJPEG()

SAVE photo with timestamp and device_id

END FOR

// Run AI analysis on captured photos

FOR EACH photo in captured_batch:

base64 = ENCODE photo AS base64

result = CALL QueryRoboflow(base64)

stage  = GROWTH_STAGE_MAP[result.predicted_class]

health = HEALTH_MAP[result.predicted_class]

predicted_days = ESTIMATE_GROWTH_TIMELINE(stage)

CALL RecordGrowthEntry(photo, stage, health, predicted_days)

END FOR

IF target_duration_reached: SET observation_active = FALSE

END WHILE

END PROCEDURE

4.3 — Roboflow AI Query

FUNCTION QueryRoboflow(base64_image):

api_url = 'https://detect.roboflow.com/' + MODEL_ID + '/' + VERSION

TRY:

response = HTTP POST api_url WITH api_key, base64_image

IF response.status == 200: RETURN PARSE response AS JSON

IF response.status == 401: RETURN ERROR 'Invalid API key'

IF response.status == 404: RETURN ERROR 'Model not found'

CATCH NetworkError: RETURN ERROR 'Network failure'

END FUNCTION

4.4 — Growth Record & Analysis

PROCEDURE RecordGrowthEntry(photo, stage, health, predicted_days):

APPEND TO plant_growth_log.csv:

{ date, time, plant_species, device_id, filename,

growth_stage, health_status, confidence,

predicted_days_remaining, roboflow_model }

EMIT socket event 'new_entry' to React Native app

END PROCEDURE

- All entries stored in plant_growth_log.csv on the Node.js server

- React Native app refreshes every 30 seconds to show latest results

- Push notifications sent when plant reaches Mature or Stressed stage

- Growth chart updates with each new entry for timeline visualization

4.5 — Program End Condition

IF target_observation_duration_achieved:

SET observation_active = FALSE

PRINT 'Growth monitoring complete for: ' + plant_species

EXPORT final_report TO plant_growth_log.csv

END PROGRAM HydroponicsMonitoringApp

Technology Stack Summary

| Component | Details |
| --- | --- |
| Hardware | Arduino Nano ESP32-S3 + External OV2640 Camera Module |
| Camera Wiring | Manual GPIO pinout — 3.3V only — 17 wire connections |
| AI / ML | Roboflow AI · YOLOv8n · 6 growth stage classes |
| Backend API | Node.js · Express.js · Multer · Socket.IO · csv-writer |
| Mobile App | React Native · Expo · React Navigation |
| Charts | Victory Native or React Native Charts Wrapper |
| Notifications | Expo Notifications · Background Fetch |
| Data Storage | plant_growth_log.csv · images/ folder on server PC |

END PROGRAM  •  HydroponicsMonitoringApp  •  ESP32-S3 Nano + React Native Edition